	[image: image1.png]GJohnson

A FAMILY COMPANY



Material Safety Data Sheet

according to ANSI Z400.1- 2004 and 29 CFR 1910.1200
	[image: image1.png]


RAID® ANT KILLER LIQUID (REG NO 18449 P.C.P. ACT)
	Version  2.0
	Print Date 07/16/2014



	Revision Date 01/10/2012  
	MSDS Number 350000011542

SITE_FORM Number 30000000000000003235.002





1. PRODUCT AND COMPANY IDENTIFICATION

Product information

	Trade name
	:
	RAID® ANT KILLER LIQUID (REG NO 18449 P.C.P. ACT)


	Use of the Substance/Mixture
	:
	Insecticide



	Company
	:
	S.C. Johnson and Son, Limited

1 Webster Street

Brantford   ON   N3T 5R1
  


	Emergency telephone number
	:
	 24 Hour Transport & Medical Emergency Phone (866) 231-5406
24 Hour International Emergency Phone (952) 852-4647
24 Hour Canadian Transport Emergency Phone (CANUTEC) (613) 996-6666




2. HAZARDS IDENTIFICATION 
Emergency Overview 
	Appearance / Odor
	:
	yellow / liquid / characteristic


	[Immediate Concerns] 

	:
	Avoid contact with skin, eyes and clothing.




Potential Health Effects 
	Exposure routes
	:
	Eye, Skin, Inhalation, Ingestion. 



	Eyes
	:
	None known. 


	Skin
	:
	May cause skin irritation. 


	Inhalation
	:
	No adverse effects expected when used as directed. 


	Ingestion
	:
	May cause irritation to mouth, throat and stomach. 
May cause abdominal discomfort. 


	Aggravated Medical Condition
	:
	Persons with pre-existing skin disorders may be more susceptible to irritating effects. 



3. COMPOSITION/INFORMATION ON INGREDIENTS

Hazardous chemicals present at or above reportable levels as defined by OSHA 29 CFR 1910.1200 or the Canadian Controlled Products Regulations are listed in this table:
	Chemical Name
	CAS-No.
	Weight percent

	Sucrose
	57-50-1
	 30.00 -  60.00


	Honey
	Mixture 
	 10.00 -  30.00


	Borax
	1303-96-4
	 5.00 -  10.00


For additional information on product ingredients, see www.whatsinsidescjohnson.com.
4. FIRST AID MEASURES

	Eye contact
	:
	Remove contact lenses. Flush immediately with plenty of water for at least 15 to 20 minutes. Get medical attention if irritation develops and persists. 



	Skin contact
	:
	Wash off immediately with plenty of water. Remove contaminated clothing and shoes. Get medical attention if irritation develops and persists. Wash clothing before reuse. Thoroughly clean shoes before reuse. 



	Inhalation
	:
	No special requirements 



	Ingestion
	:
	If swallowed, DO NOT induce vomiting unless directed to do so by medical personnel. Never give anything by mouth to an unconscious person. Get medical attention immediately. 




5. FIREFIGHTING MEASURES

	Suitable extinguishing media
	:
	Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide.


	Specific hazards during firefighting
	:
	Container may melt and leak in heat of fire. Do not allow run-off from fire fighting to enter drains or water courses. 


	Further information
	:
	Fight fire from maximum distance or protected area. Wear full protective clothing and positive pressure self-contained breathing apparatus. In case of fire and/or explosion do not breathe fumes. 


	Flash point
	:
	Note: no data available




	Lower explosion limit
	:
	Note: no data available




	Upper explosion limit
	:
	Note: no data available




6. ACCIDENTAL RELEASE MEASURES

	Personal precautions
	:
	Wear personal protective equipment.



	Environmental precautions
	:
	Do not flush into surface water or sanitary sewer system.

Use appropriate containment to avoid environmental contamination.

Outside of normal use, avoid release to the environment.



	Methods for cleaning up
	:
	Soak up with inert absorbent material.

Sweep up and shovel into suitable containers for disposal.

Dike large spills.

Clean residue from spill site.




7. HANDLING AND STORAGE

Handling
	Advice on safe handling
	:
	Avoid contact with skin, eyes and clothing.

Do not enter places where used or stored until adequately ventilated.

For personal protection see section 8.

Use only as directed.

KEEP OUT OF REACH OF CHILDREN AND PETS.



	Advice on protection against fire and explosion
	:
	Normal measures for preventive fire protection.



Storage
	Requirements for storage areas and containers
	:
	Keep away from food, drink and animal feedingstuffs.

Keep container closed when not in use.

Keep in a dry, cool and well-ventilated place.




8. EXPOSURE CONTROLS/PERSONAL PROTECTION

Occupational Exposure Limits

	Components
	CAS-No.
	mg/m3

 
	ppm 


	Non-standard units
	Basis

	Sucrose
	57-50-1
	10 mg/m3

	-


	-


	ACGIH TWA


	Borax
	1303-96-4
	6 mg/m3

	-


	-


	ACGIH STEL


	Borax
	1303-96-4
	2 mg/m3

	-


	-


	ACGIH TWA



Personal protective equipment
	Respiratory protection

	
	

	Industrial setting
	:
	No personal respiratory protective equipment normally required.



	Household setting
	:
	Use only with adequate ventilation.

	
	
	

	Hand protection

	:
	If prolonged or repeated contact is possible:
Wear suitable gloves.



	Eye protection

	
	

	Industrial setting
	:
	No special requirements.



	Household setting
	:
	Avoid contact with eyes.

	
	
	

	Skin and body protection

	:
	No special requirements.



	Hygiene measures
	:
	Handle in accordance with good industrial hygiene and safety practice. Wash thoroughly after handling. 




9. PHYSICAL AND CHEMICAL PROPERTIES

	Form
	:
	liquid


	Color
	:
	yellow


	Odor
	:
	characteristic



	pH 
	:
	4.0 - 5.0


	Melting point
	:
	no data available



	Boiling point
	:
	no data available



	Freezing point
	:
	no data available


	Flash point
	:
	no data available



	Evaporation rate
	:
	no data available



	Autoignition temperature 
	:
	no data available



	Lower explosion limit

	:
	no data available



	Upper explosion limit
	:
	no data available



	Vapour pressure
	:
	no data available



	Density
	:
	1.23 - 1.25 g/cm3


	Water solubility
	:
	completely soluble



	Partition coefficient: n-octanol/water
	:
	no data available




10. STABILITY AND REACTIVITY

	Conditions to avoid
	:
	Direct sources of heat.
  

	Materials to avoid
	:
	None known.


	Hazardous decomposition products
	:
	Thermal decomposition can lead to release of irritating gases and vapours.



	Hazardous reactions
	:
	Stable under recommended storage conditions.



11. TOXICOLOGICAL INFORMATION

	Acute oral toxicity
	:
	LD50

Calculated
4,294 mg/kg



	Acute inhalation toxicity
	:
	LC50
Calculated

2.00 mg/l



	Acute dermal toxicity
	:
	LD50
Calculated
9,853 mg/kg



Chronic effects

	Carcinogenicity
	:
	no data available




	Mutagenicity
	:
	None Anticipated


	Reproductive effects
	:
	no data available




	Teratogenicity
	:
	no data available




	Sensitisation
	:
	no data available


12. ECOLOGICAL INFORMATION

	Ecotoxicity effects
	:
	no data available



13. DISPOSAL CONSIDERATIONS

	
	
	Observe all applicable Federal, Provincial and State regulations and Local/Municipal ordinances regarding disposal.
Consumer may discard empty container in trash, or recycle where facilities exist.




14. TRANSPORT INFORMATION
Please refer to the Bill of Lading/receiving documents for up-to-date shipping information.
Land transport
· U.S. DOT and Canadian TDG Surface Transportation:

	Proper shipping name
	not regulated

	Class:
	None. 

	UN/ID No.:
	None.  

	Packaging group
	None.


Sea transport

· IMDG:
	Proper shipping name
	not regulated

	Class:
	None. 

	UN/ID No.:
	None.  

	Packaging group
	None.


Air transport

· ICAO/IATA:

	Proper shipping name
	not regulated

	Class:
	None. 

	UN/ID No.:
	None.  

	Packaging group
	None.


15. REGULATORY INFORMATION

	Notification status
	:
	All ingredients of this product comply with the New Substances Notification requirements under the Canadian Environmental Protection Act (CEPA).



	California Prop. 65
	:
	


	
	:
	This product has been classified in accordance with hazard criteria of the Controlled Products Regulations and the MSDS contains all the information required by the Controlled Products Regulations.



16. OTHER INFORMATION


HMIS Ratings 

	Health
	1



	Flammability
	0


	Reactivity
	0




NFPA Ratings 

	Health
	1


	Fire
	0


	Reactivity
	0


	Special
	-




Further information
This document has been prepared using data from sources considered to be technically reliable. It does not constitute a warranty, expressed or implied, as to the accuracy of the information contained herein. Actual conditions of use are beyond the seller's control. User is responsible to evaluate all available information when using product for any particular use and to comply with all Federal, State, Provincial and Local laws and regulations.
	Prepared by:
	SC Johnson Global Safety Assessment & Regulatory Affairs (GSARA)


1/8

