1. PRODUCT AND COMPANY IDENTIFICATION

Product information

| Trade name | : | GLADE® SOLID AIR FRESHENER - APPLE CINNAMON |
| --- | --- | --- |
| Use of the Substance/Mixture | : | Air Freshener |
| Company | : | S.C. Johnson and Son, Limited 1 Webster Street Brantford ON N3T 5R1 |

| Emergency telephone number | : | 24 Hour Transport & Medical Emergency Phone (866) 231-5406 24 Hour International Emergency Phone (952) 852-4647 24 Hour Canadian Transport Emergency Phone (CANUTEC) (613) 996-6666 |
| --- | --- | --- |

2. HAZARDS IDENTIFICATION

Emergency Overview

| Appearance / Odor | : | Burgundy / gel / pleasant |
| --- | --- | --- |

| [Immediate Concerns] | : | Avoid contact with skin, eyes and clothing. |
| --- | --- | --- |

Potential Health Effects

| Exposure routes | : | Eye, Skin, Inhalation, Ingestion. |
| --- | --- | --- |
| Eyes | : | May cause: Mild eye irritation |
| Skin | : | May cause skin reactions in rare cases. Prolonged or repeated contact may dry skin and cause irritation. |
| Inhalation | : | No adverse effects expected when used as directed. |
| Ingestion | : | May cause irritation to mouth, throat and stomach. May cause abdominal discomfort. |
| Aggravated Medical Condition | : | Persons with pre-existing skin disorders may be more susceptible to irritating effects. |

3. COMPOSITION/INFORMATION ON INGREDIENTS

Hazardous chemicals present at or above reportable levels as defined by OSHA 29 CFR 1910.1200 or the Canadian Controlled Products Regulations are listed in this table:

| Chemical Name | CAS-No. | Weight percent |
| --- | --- | --- |
| Carrageenan | 9000-07-1 | 1.00 - 5.00 |

| Fragrance - Trade Secret | Mixture | 1.00 - 5.00 |
| --- | --- | --- |

For additional information on product ingredients, see www.whatsinsidescjohnson.com.

4. FIRST AID MEASURES

| Eye contact | : | Rinse with plenty of water. Get medical attention if irritation develops and persists. |
| --- | --- | --- |
| Skin contact | : | Rinse with plenty of water. Get medical attention if irritation develops and persists. |
| Inhalation | : | No special requirements |
| Ingestion | : | Rinse mouth with water. |

5. FIREFIGHTING MEASURES

| Suitable extinguishing media | : | Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide. |
| --- | --- | --- |

| Further information | : | Fight fire with normal precautions from a reasonable distance. Standard procedure for chemical fires. Wear full protective clothing and positive pressure self-contained breathing apparatus. |
| --- | --- | --- |

| Flash point | : | Note: does not flash |
| --- | --- | --- |

| Lower explosion limit | : | Note: no data available |
| --- | --- | --- |

| Upper explosion limit | : | Note: no data available |
| --- | --- | --- |

6. ACCIDENTAL RELEASE MEASURES

| Personal precautions | : | No special precautions required. |
| --- | --- | --- |
| Environmental precautions | : | Outside of normal use, avoid release to the environment. |
| Methods for cleaning up | : | Sweep up and shovel into suitable containers for disposal. Clean residue from spill site. |

7. HANDLING AND STORAGE

Handling

| Advice on safe handling | : | Use only as directed. KEEP OUT OF REACH OF CHILDREN AND PETS. |
| --- | --- | --- |
| Advice on protection against fire and explosion | : | Normal measures for preventive fire protection. |

Storage

| Requirements for storage areas and containers | : | Keep in a dry, cool and well-ventilated place. |
| --- | --- | --- |

8. EXPOSURE CONTROLS/PERSONAL PROTECTION

Occupational Exposure Limits

ACGIH or OSHA exposure limits have not been established for this product or reportable ingredients unless noted in the table above.

Personal protective equipment

| Respiratory protection |  | No personal respiratory protective equipment normally required. |
| --- | --- | --- |
| Hand protection | : | No special requirements. |
| Eye protection |  | No special requirements. |
| Skin and body protection | : | No special requirements. |
| Hygiene measures | : | Handle in accordance with good industrial hygiene and safety practice. Wash thoroughly after handling. |

9. PHYSICAL AND CHEMICAL PROPERTIES

| Form | : | gel |
| --- | --- | --- |
| Color | : | Burgundy |
| Odor | : | pleasant |

| pH | : | no data available |
| --- | --- | --- |
| Melting point | : | no data available |
| Boiling point | : | no data available |
| Freezing point | : | 33 °F |
| Flash point | : | does not flash |
| Evaporation rate | : | no data available |
| Flammability (solid, gas) | : | no data available |
| Auto-ignition temperature | : | no data available |
| Lower explosion limit | : | no data available |
| Upper explosion limit | : | no data available |
| Vapour pressure | : | no data available |
| Density | : | > 1.0 g/cm3 |
| Water solubility | : | no data available |
| Partition coefficient: n-octanol/water | : | no data available |
| Viscosity, dynamic | : | no data available |
| Viscosity, kinematic | : | no data available |
| Relative vapour density | : | no data available |

| Volatile Organic Compounds Total VOC (wt. %)* | : | 1.2 % - additional exemptions may apply *as defined by US Federal and State Consumer Product Regulations |
| --- | --- | --- |

10. STABILITY AND REACTIVITY

| Conditions to avoid | : | Direct sources of heat. |
| --- | --- | --- |
| Materials to avoid | : | None known. |
| Hazardous decomposition products | : | Thermal decomposition can lead to release of irritating gases and vapours. |

| Thermal decomposition | : | Note: no data available |
| --- | --- | --- |
| Hazardous reactions | : | Stable under recommended storage conditions. |

# 11. TOXICOLOGICAL INFORMATION

| Acute oral toxicity | : | LD50 estimated > 5,000 mg/kg |
| --- | --- | --- |

| Acute inhalation toxicity | : | LC50 estimated > 2 mg/l |
| --- | --- | --- |

| Acute dermal toxicity | : | LD50 estimated > 2,000 mg/kg |
| --- | --- | --- |

Chronic effects

| Carcinogenicity | : | no data available |
| --- | --- | --- |

| Mutagenicity | : | no data available |
| --- | --- | --- |

| Reproductive effects | : | no data available |
| --- | --- | --- |

| Teratogenicity | : | no data available |
| --- | --- | --- |

| Sensitisation | : | Not known to be a sensitizer. |
| --- | --- | --- |

12. ECOLOGICAL INFORMATION

| Ecotoxicity effects | : | no data available |
| --- | --- | --- |

13. DISPOSAL CONSIDERATIONS

|  |  | Observe all applicable Federal, Provincial and State regulations and Local/Municipal ordinances regarding disposal. Consumer may discard empty container in trash, or recycle where facilities exist. |
| --- | --- | --- |

14. TRANSPORT INFORMATION

Land transport

- U.S. DOT and Canadian TDG Surface Transportation:

| Proper shipping name | not regulated |
| --- | --- |
| Class: | None. |
| UN/ID No.: | None. |
| Packaging group | None. |

Sea transport

- IMDG:

| Proper shipping name | not regulated |
| --- | --- |
| Class: | None. |
| UN/ID No.: | None. |
| Packaging group | None. |

Air transport

- ICAO/IATA:

| Proper shipping name | not regulated |
| --- | --- |
| Class: | None. |
| UN/ID No.: | None. |
| Packaging group | None. |

15. REGULATORY INFORMATION

| Notification status | : | All ingredients of this product are listed or are excluded from listing on the U.S. Toxic Substances Control Act (TSCA) Chemical Substance Inventory. |
| --- | --- | --- |
| Notification status | : | All ingredients of this product comply with the New Substances Notification requirements under the Canadian Environmental Protection Act (CEPA). |
| California Prop. 65 | : | This product does not contain any chemicals known to State of California to cause cancer, birth defects, or any other reproductive harm. |
| Canada Regulations | : | This product has been classified in accordance with hazard criteria of the Controlled Products Regulations and the MSDS contains all the information required by the Controlled Products Regulations. |

16. OTHER INFORMATION

HMIS Ratings

| Health | 1 |
| --- | --- |
| Flammability | 0 |
| Reactivity | 0 |

NFPA Ratings

| Health | 1 |
| --- | --- |
| Fire | 0 |
| Reactivity | 0 |
| Special | - |

This information is being provided in accordance with Occupational Safety and Health Administration (OSHA) and Canada’s Workplace Hazard Material Information System (WHMIS) regulations. The information supplied is designed for workplaces where product use and frequency of exposure exceeds that established for the labeled consumer use.

Further information

This document has been prepared using data from sources considered to be technically reliable. It does not constitute a warranty, expressed or implied, as to the accuracy of the information contained herein. Actual conditions of use are beyond the seller's control. User is responsible to evaluate all available information when using product for any particular use and to comply with all Federal, State, Provincial and Local laws and regulations.

| Prepared by | SC Johnson Global Safety Assessment & Regulatory Affairs (GSARA) |
| --- | --- |