1. PRODUCT AND COMPANY IDENTIFICATION

Product information

| Trade name | : | GLADE PLUGINS® SCENTED OIL - HAWAIIAN BREEZE® |
| --- | --- | --- |
| Use of the Substance/Mixture | : | Air Freshener |
| Company | : | S.C. Johnson and Son, Limited 1 Webster Street Brantford ON N3T 5R1 |

| Emergency telephone number | : | 24 Hour Transport & Medical Emergency Phone (866) 231-5406 24 Hour International Emergency Phone (952) 852-4647 24 Hour Canadian Transport Emergency Phone (CANUTEC) (613) 996-6666 |
| --- | --- | --- |

2. HAZARDS IDENTIFICATION

Emergency Overview

| Appearance / Odor | : | yellow / liquid / pleasant |
| --- | --- | --- |

| [Immediate Concerns] | : | Caution COMBUSTIBLE: CAUSES EYE IRRITATION. Avoid contact with skin, eyes and clothing. Keep away from heat, sparks and flame. |
| --- | --- | --- |

Potential Health Effects

| Exposure routes | : | Eye, Skin, Inhalation, Ingestion. |
| --- | --- | --- |
| Eyes | : | Causes: Moderate eye irritation |
| Skin | : | May cause skin reactions in rare cases. Prolonged or repeated contact may dry skin and cause irritation. |
| Inhalation | : | No adverse effects expected when used as directed. |
| Ingestion | : | May cause irritation to mouth, throat and stomach. May cause abdominal discomfort. |
| Aggravated Medical Condition | : | Persons with pre-existing skin disorders may be more susceptible to irritating effects. |

3. COMPOSITION/INFORMATION ON INGREDIENTS

Hazardous chemicals present at or above reportable levels as defined by OSHA 29 CFR 1910.1200 or the Canadian Controlled Products Regulations are listed in this table:

| Chemical Name | CAS-No. | Weight percent |
| --- | --- | --- |
| Fragrance - Trade Secret | Mixture | 60.00 - 100.00 |

For additional information on product ingredients, see www.whatsinsidescjohnson.com.

4. FIRST AID MEASURES

| Eye contact | : | Remove contact lenses. Flush immediately with plenty of water for at least 15 to 20 minutes. Get medical attention if irritation develops and persists. |
| --- | --- | --- |
| Skin contact | : | Rinse with plenty of water. Get medical attention if irritation develops and persists. |
| Inhalation | : | No special requirements |
| Ingestion | : | If swallowed, DO NOT induce vomiting unless directed to do so by medical personnel. Never give anything by mouth to an unconscious person. Get medical attention immediately. |

5. FIREFIGHTING MEASURES

| Suitable extinguishing media | : | Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide. |
| --- | --- | --- |

| Specific hazards during firefighting | : | Do not use a solid water stream as it may scatter and spread fire. Container may melt and leak in heat of fire. |
| --- | --- | --- |

| Further information | : | Cool and use caution when approaching or handling fire-exposed containers. Wear full protective clothing and positive pressure self-contained breathing apparatus. In case of fire and/or explosion do not breathe fumes. |
| --- | --- | --- |

| Flash point | : | 71 °C 159.8 °F |
| --- | --- | --- |

| Lower explosion limit | : | Note: no data available |
| --- | --- | --- |

| Upper explosion limit | : | Note: no data available |
| --- | --- | --- |

6. ACCIDENTAL RELEASE MEASURES

| Personal precautions | : | Remove all sources of ignition. Beware of vapours accumulating to form explosive concentrations. Vapours can accumulate in low areas. Wear personal protective equipment. |
| --- | --- | --- |
| Environmental precautions | : | Outside of normal use, avoid release to the environment. |
| Methods for cleaning up | : | Contain spillage, soak up with non-combustible absorbent material, (e.g. sand, earth, diatomaceous earth, vermiculite) and transfer to a container for disposal according to local / national regulations (see section 13). Use only non-sparking equipment. Dike large spills. Clean residue from spill site. |

7. HANDLING AND STORAGE

Handling

| Advice on safe handling | : | Avoid contact with skin, eyes and clothing. For personal protection see section 8. Use only as directed. KEEP OUT OF REACH OF CHILDREN AND PETS. |
| --- | --- | --- |
| Advice on protection against fire and explosion | : | Keep away from heat and sources of ignition. Take measures to prevent the build up of electrostatic charge. |

Storage

| Requirements for storage areas and containers | : | Keep container closed when not in use. Keep in a dry, cool and well-ventilated place. |
| --- | --- | --- |

| Storage Stability | : | Stable under normal conditions. |
| --- | --- | --- |

8. EXPOSURE CONTROLS/PERSONAL PROTECTION

Occupational Exposure Limits

ACGIH or OSHA exposure limits have not been established for this product or reportable ingredients unless noted in the table above.

Personal protective equipment

| Respiratory protection |  | No personal respiratory protective equipment normally required. |
| --- | --- | --- |
| Hand protection | : | No special requirements. |
| Eye protection |  | Safety glasses with side-shields |
| Skin and body protection | : | No special requirements. |
| Hygiene measures | : | Handle in accordance with good industrial hygiene and safety practice. Wash thoroughly after handling. |

9. PHYSICAL AND CHEMICAL PROPERTIES

| Form | : | liquid |
| --- | --- | --- |
| Color | : | yellow |
| Odor | : | pleasant |

| pH | : | no data available |
| --- | --- | --- |
| Boiling point | : | not applicable |
| Freezing point | : | not applicable |
| Flash point | : | 71 °C 159.8 °F |
| Evaporation rate | : | not applicable |
| Flammability (solid, gas) | : | Sustains combustion |
| Lower explosion limit | : | no data available |
| Upper explosion limit | : | no data available |
| Vapour pressure | : | no data available |
| Density | : | 0.9580 g/ml at 25 °C |
| Water solubility | : | negligible |
| Viscosity, dynamic | : | no data available |
| Viscosity, kinematic | : | no data available |
| Volatile Organic Compounds Total VOC (wt. %)* | : | 69.6 % - additional exemptions may apply *as defined by US Federal and State Consumer Product Regulations |

10. STABILITY AND REACTIVITY

| Conditions to avoid | : | Heat, flames and sparks. |
| --- | --- | --- |
| Materials to avoid | : | Strong oxidizing agents |
| Hazardous decomposition products | : | Thermal decomposition can lead to release of irritating gases and vapours. |

| Hazardous reactions | : | Stable under recommended storage conditions. |
| --- | --- | --- |

# 11. TOXICOLOGICAL INFORMATION

| Acute oral toxicity | : | LD50 estimated > 5,000 mg/kg |
| --- | --- | --- |

| Acute inhalation toxicity | : | LC50 estimated > 2 mg/l |
| --- | --- | --- |

| Acute dermal toxicity | : | LD50 estimated > 2,000 mg/kg |
| --- | --- | --- |

Chronic effects

| Carcinogenicity | : | no data available |
| --- | --- | --- |

| Mutagenicity | : | no data available |
| --- | --- | --- |

| Reproductive effects | : | no data available |
| --- | --- | --- |

| Teratogenicity | : | no data available |
| --- | --- | --- |

| Sensitisation | : | Not known to be a sensitizer. |
| --- | --- | --- |

12. ECOLOGICAL INFORMATION

| Ecotoxicity effects | : | no data available |
| --- | --- | --- |

13. DISPOSAL CONSIDERATIONS

|  |  | Observe all applicable Federal, Provincial and State regulations and Local/Municipal ordinances regarding disposal. Consumer may discard empty container in trash, or recycle where facilities exist. |
| --- | --- | --- |
| RCRA waste class | : | D001 (Ignitable Waste) |  |

14. TRANSPORT INFORMATION

Please refer to the Bill of Lading/receiving documents for up-to-date shipping information.

Land transport

- U.S. DOT and Canadian TDG Surface Transportation:

| Proper shipping name | not regulated |
| --- | --- |
| Class: | None. |
| UN/ID No.: | None. |
| Packaging group | None. |

Sea transport

- IMDG:

| Proper shipping name | not regulated |
| --- | --- |
| Class: | None. |
| UN/ID No.: | None. |
| Packaging group | None. |

Air transport

- ICAO/IATA:

| Proper shipping name | not regulated |
| --- | --- |
| Class: | None. |
| UN/ID No.: | None. |
| Packaging group | None. |

15. REGULATORY INFORMATION

| Notification status | : | All ingredients of this product are listed or are excluded from listing on the U.S. Toxic Substances Control Act (TSCA) Chemical Substance Inventory. |
| --- | --- | --- |
| Notification status | : | All ingredients of this product comply with the New Substances Notification requirements under the Canadian Environmental Protection Act (CEPA). |
| California Prop. 65 | : | This product is not subject to the reporting requirements under California's Proposition 65. |
| Canada Regulations | : | This product has been classified in accordance with hazard criteria of the Controlled Products Regulations and the MSDS contains all the information required by the Controlled Products Regulations. |

16. OTHER INFORMATION

HMIS Ratings

| Health | 2 |
| --- | --- |
| Flammability | 2 |
| Reactivity | 0 |

NFPA Ratings

| Health | 2 |
| --- | --- |
| Fire | 2 |
| Reactivity | 0 |
| Special | - |

This information is being provided in accordance with Occupational Safety and Health Administration (OSHA) and Canada’s Workplace Hazard Material Information System (WHMIS) regulations. The information supplied is designed for workplaces where product use and frequency of exposure exceeds that established for the labeled consumer use.

Further information

This document has been prepared using data from sources considered to be technically reliable. It does not constitute a warranty, expressed or implied, as to the accuracy of the information contained herein. Actual conditions of use are beyond the seller's control. User is responsible to evaluate all available information when using product for any particular use and to comply with all Federal, State, Provincial and Local laws and regulations.

| Prepared by | SC Johnson Global Safety Assessment & Regulatory Affairs (GSARA) |
| --- | --- |