Non-Aerosol Products Include:

Prince Matchabelli Colognes:               Aviance Night Musk 	    New Musk for Women

SIZES ABOVE 1 OZ                             Cachet			    Skin Musk Cologne/Oil

(Glass Bottles)			             Cotton Candy		    Wind Song

Fresh White Musk              Sexiest Musk

ALL Men’s PDC/Designer Imposters Colognes/BOD Cologne:

(Glass Bottles)				Being Together		    Mascolino

Fresh Xtreme		    New York Nights

Tahoe			    U

Bring It		    	    U Two

Smokin’ Hot		    Big Attitude

KING			    Body Heat

Rule                                 Really Ripped Abs on Steroids

Raw Power		   Game Changer

Body Fantasies/Signature Fantasies Body Sprays:

(Plastic Bottles)	   		Apple			    Sexiest Musk

Cherry Blossom		    Paradise

Cotton Candy		    Clean Lavender Vanilla

Clean Ocean Air                 Japanese Cherry Blossom

Cucumber Melon	    Iced Cupcake

Freesia			    Fresh White Musk

Pear			    Twilight Mist

Plumeria		    Sugar Apple

Raspberry		    Sugar Vanilla

Vanilla			    Coconut Breeze

Pasion Tropicale	    Fiesta Floral

Feel Elegant		    Lemon Sugar

Rose Petals		    Pink Sweet Pea

Sweet Sunrise		    Pink Vanilla Kiss

Silver Linings		    Romance & Dreams

Strawberry

Sexiest Fantasies Body Sprays:	Fireworks		    Love Struck

(Plastic Bottles)			On The Prowl		    Slow Dance

Strawberry & Champagne  VaVaVoom

Elegant Night Musk            Midnight Oud

Oud Noir		     Japanese Cherry Blossom

Tempt Me Sweetly

Body Fantasies Cologne:		Vampire

(Glass Bottles)

BF Luxe Pump Cologne: (Int’l)		Fleur Joyeuse

Paradise in Bloom

Sparkling Breeze

Juice Bar Body Sprays:		Gummi Bears		     Cotton Candy

Fresh & Pretty Body Spray:		Apple			     Cucumber Melon

(Plastic Bottles)			Pear		                  Raspberry

White Musk

BOD Body Sprays:			Fresh Blue Musk	      X

(Plastic Bottles, Trio Sets)		Really Ripped Abs	     Player

Rock Hard		     Fresh Guy

Tekno			     Black

Silver  		                  24K

Money			     Freshest Cleanest

Most Wanted	                  Warrior

Lights Out		     Blue Surf

Midnight Reign

DI – Womens 1.8 oz Pump Cologne:	Primo			     A Little Sexy

(Glass Bottles)

DI – Womens/Mens

Pump Body Sprays:	     Swept Away          Wanna Play                  Raw Power

(Plastic Bottles/trio sets)   Primo	          New York Nights         Mascolino

Bring It                   Smokin’ Hot                Game Changer

# MATERIAL SAFETY DATA SHEET

# NON-AEROSOL PRODUCTS

# SECTION I		IDENTIFICATION

Manufacturer:			Pharmasol Corporation

Telephone No. 			508-238-8501

Address:				1 Norfolk Ave. So. Easton MA 02375

Product Name:/Trade Name:	Non-aerosol

Formula:			This product is a proprietary mixture

Date:				May 29, 2003, Rev. 3

# SECTION II		COMPOSITION INFORMATION

## Chemical Name		CAS Number		Percent by wgt.	OSHA PEL

Ethanol			64-17-5		60%-85%		1000 ppm

Fragrance Oil		mixture		2% - 20%		Not Established

Propylene glycol		41395-83-9	5%-20%			Not established

Water			7732-18-5	5% - 20%		Not Established

Dyes			Mixture		< 2%			Not established

# SECTION III		HAZARDS IDENTIFICATION AND CHEMICAL PROPERTIES

Physical State:		Liquid		Appearance:		Slightly Colored, Solvent

pH:			Not available	Solubility in water:	Soluble

Odor:			Fragrant		Flash Point:		Mixture, not established

Percent Volatiles:		100 % (W)	Boiling Point:		Mixture, not established

Freezing Point:	Mixture N/A		Specific Gravity:		Less than water

Vapor Pressure:	Mixture N/A		Vapor Density:		Heavier than air

Hazards:			Flammable mixture

Potential Health Effects:

Inhalation:	May cause a burning sensation in the nose and throat, and stinging and watering in the eyes. At concentrations which cause irritation, dizziness, faintness, drowsiness, nausea, and vomiting

Eye Contact:	May cause irritation, experienced as stinging with excess blinking and tear production.

Skin Contact:	Redness, dryness and contact or allergic dermatitis may occur in sensitive individuals.

Skin Absorption:	No harmful effects with normal skin

Ingestion:	May cause dizziness, faintness, drowsiness, decreased awareness and responsiveness, euphoria, abdominal discomfort, nausea, vomiting, staggering gait, lack of coordination, and coma.

Chronic, Prolonged or Repeated Overexposure:

Long term repeated oral exposure to ethanol may result in the development of progressive liver injury with fibrosis. Repeated ingestion of ethanol by pregnant mothers has been shown to adversely affect the central nervous system of the fetus, producing a collection of effects which together constitute the fetal alcohol syndrome. These include mental and physical retardation, disturbances of learning, motor and language deficiencies, behavioral disorders, and small size head.

###### SECTION IV 	FIRST AID PROCEDURES

Inhalation:	Remove to fresh air. Give artificial respiration if not breathing. If breathing is difficult, oxygen my be given by qualified personnel. Obtain medical attention.

Eye Contact:	Flush eyes with water and continue washing for several minutes.

Skin Contact:	Wash skin with soap and water.

Swallowing:	If patient is fully conscious, give two glasses of water. Induce vomiting.

NON-AEROSOL – PAGE 2

# SECTION V		FIRE FIGHTING MEASURES

Flash Point:	(Ethanol)	17.2 degrees C, 63 degrees F. (Tag Closed Cup)

Auto-ignition Temperature:		Not available

Flammable limits in air (Ethanol)	LEL	3.3% (V), UEL	19.0%  (V)

(This substance is a mixture and the LEL/UEL range is not established)

Extinguishing Media:

Apply alcohol-type or all-purpose-type foam by manufacturer’s recommended techniques for large fires. Use carbon dioxide or dry chemical media for small fires.  Do not use Type A (water) fire extinguisher on flammable liquids fires.

# SECTION VI		ACCIDENTAL RELEASE MEASURES

Steps to be taken if Material is Released or Spilled:

Extinguish and do not turn on any ignition source until the area is determined to be free from fire or explosion hazard. Small spills can be flushed with large amounts of water, larger spills should be collected for disposal.  Observe government regulations. Wear suitable personal protective equipment.

# SECTION VII	HANDLING AND STORAGE

Keep away from heat, sparks and flame. Avoid contact with eyes. Keep container closed. Use with adequate ventilation. Vapors from this material may settle in low or confined areas or travel a long distance to an ignition source and flash back explosively. Wash thoroughly after handling.

# SECTION VIII		STABILITY AND REACTIVITY

Stability:			Stable

Incompatible Materials:	Concentrated nitric and sulfuric acids, strong oxidizing agents.

Hazardous Polymerization will not occur.

### SECTION IX  TOXICOLOGICAL INFORMATION

Non-beverage uses of pure ethanol are not considered to pose any significant cancer hazard. Ethanol has been shown to have a weak skin sensitizing potential in a very small percentage of the population.

# SECTION X	DISPOSAL CONSIDERATION

Incinerate in a furnace where permitted under Federal, State and local regulations. Dispose in accordance with all applicable Federal, State, Provincial, and local environmental regulations. Empty containers should be recycled or disposed of through an approved waste management facility.

# SECTION XI	TRANSPORT INFORMATION

U.S. D.O.T.

## Non-Bulk

Proper shipping name:	Consumer Commodity / ORM

This information is not intended to convey all specific regulatory or operational requirements/information relating to this product. Additional transportation system information can be obtained through your Pharmasol sales or customer service representative. It is the responsibility of the transporting organization to follow all the applicable laws, regulations and rules relating to the transportation of the material.

# SECTION XII	OTHER INFORMATION

Additional information about product safety may be obtained by calling Pharmasol Corporation at

1-508-238-8501